From: stuce@csv.warwick.ac.uk (Peter Weighill)

Newsgroups: comp.sys.cbm

Subject: 1541 disk drive commands.

Date: 25 Sep 1993 19:20:47 +0100

Organization: Commodore 64 Services, University of Warwick, UK

TWO RARELY USED 1541 DISK DRIVE FUNCTIONS

1. The Utility Loader '&' command.

INTRODUCTION

A little known and little used command on the 1541 disk drive is the

"&" command. This is probably due to the fact that there is no mention of it

in the "1541 DISK DRIVE users guide". Many other books about the disk drive

also fail to mention it. As to a use for the command, I have not found one

yet. Perhaps someone could think of one. I would expect that the 1570/1571

and 1581 drive will also contain the "&" command as well, since they are

based on the 1541.

UTILITY LOADER  ("&" command)

The utility loader is the command which will load a USR file from

disk into disk drive memory where it will then execute. The format for

the command is as follows:

OPEN15,8,15:PRINT#15,"&filename":CLOSE15

USR FILES

A user file has to follow certain guidlines. It is limited to just

one sector and this sector is constructed as below;

Byte

0      Start address low  order

1      Start address high order

2      Number of bytes in program

3+     Program code bytes

last   Checksum

This means that the maximum size of program code is 251 bytes. The

checksum byte is calculated by adding all the values of the bytes, starting

at the low order start address (byte 0), while adding you subtract 255 from

the total every time it exceeds 255. Thus the checksum byte can range from

1-255.

One final constraint is that the filename must begin with an "&".

Below is a program which will make it easier for you to create a USR

file in the required format, so that it can be executed by the utility

loader command. The program automatically calculates the length of the code

and also the checksum at the end. All you need to do is add your own code

to the data statements between 210 and 300 and specify a filename in line 10.

10  OPEN2,8,2,"0:&filename,U,W"

20  READLO,HI:C=LO+HI:IFC>255THENC=C-255

30  PRINT#2,CHR$(LO)CHR$(HI);

40  T$=""

50  READD:IFD=-1THEN100

60  C=C+D:IFC>255THENC=C-255

70  T$=T$+CHR$(D)

80  GOTO50

100 L=LEN(T$)

110 C=C+L:IFC>255THENC=C-255

120 PRINT#2,CHR$(L)T$CHR$(C);

130 CLOSE2

140 END

170 :

200 DATA 0,5 :REM lo/hi start address

210 :         REM program code

220 DATA 173,0,28,41,16,201,16,208,11

230 DATA 169,247,45,0,28,141,0,28,76

240 DATA 0,5,32,24,193,76,0,5

300 DATA -1  :REM program data finished

The example code in the program is not that useful, it is just there

to show how the utility loader works. It just switches the drives light on

and off depending on if the write protect sensor is covered or not.

If you can think of something which could be done with the utility

loader then I would love to hear of it.

ERRORS THAT CAN OCCUR

39, 'file not found'

This occurs if the file you specified using the utility loader

command does not exist or is not a USR file.

50, 'record not present'

The checksum calculated by the disk drive and the checksum at the

end of the file differ.

51, 'overflow in record'

Either: 1. The end of the file was reached before the checksum

byte was read.

or      2. The file contains extra bytes of data after the checksum

byte.

Check the length byte in the usr file (position 3, after lo/hi.)

2. Checking that a file is on the disk.

INTRODUCTION

If you wrote a program which needed to check that a particular file

existed on a disk then you would probably open the file for a read, then

check the error channel for 62, FILE NOT FOUND.

Thus:

10 OPEN15,8,15

20 OPEN2,8,2,"filename,P,R"

30 INPUT#15,E,E$

40 IFE>0THENPRINTE$:GOTO60

50 PRINT"FILE EXISTS"

60 CLOSE2:CLOSE15

Another way to check if a file exists is to try to rename it as itself.

Thus:

10 OPEN15,8,15,"R:filename=filename"

20 INPUT#15,E,E$

30 CLOSE15

40 PRINTE$

If the file exists then the error created is 63, FILE EXISTS,

otherwise it is 62, FILE NOT FOUND. Judge for yourself which works better.

Written, September 93, by Peter Weighill.   email: stuce@warwick.csv.ac.uk

From: Nicholas.Cull@comp.vuw.ac.nz (Nicholas Cull)

Newsgroups: comp.sys.cbm

Subject: Re: 1541 disk drive commands.

Date: 30 Sep 1993 00:00:20 GMT

Organization: Dept. of Comp. Sci., Victoria Uni. of Wellington, New Zealand.

Originator: ncull@comp.vuw.ac.nz

In article <28225v$1s3@tansy.csv.warwick.ac.uk>, stuce@csv.warwick.ac.uk

(Peter Weighill) writes:

|>

|> 1. The Utility Loader '&' command.

...

|>      A user file has to follow certain guidlines. It is limited to just

^^^^^^^^^^^^^^^^^^^^^

|> one sector and this sector is constructed as below;

^^^^^^^^^^

This is incorrect. The user file may be more than one sector long, although the

maximum size of the program code is limited to 256 bytes. To obtain this, byte

2 (the number of bytes in program) should be set to 0. A formula for this

would be

(NB AND 255)

where NB is the number of bytes. Remember that the checksum has to be updated

in a similar way too, eg

C = C+(NB AND 255): IF C>255 THEN C=C-255

The number of bytes in a Block-Execute command is limited to a single sector,

which is 256 bytes, although these can be all data bytes. Maybe you were

confusing the two commands?

|>

|> Byte

|> 0      Start address low  order

|> 1      Start address high order

|> 2      Number of bytes in program

|> 3+     Program code bytes

|> last   Checksum

|>

In fact there can be more than one block of this construction per file. To do

this, simply repeat the construction. For example, if you wanted to have a

program which sat in drive locations $0300 to $0400, and $0480 to $0500, the

construction would be as follows:

Byte

0             Low byte start address of first block (0)

1             High byte start address of first block (3)

2             Number of bytes in this block (0)        [= 256 AND 255]

3 - 259       Program code for first block (data goes in here...)

260           Checksum for first block (whatever this may be)

261           Low byte start address of second block (128)

262           High byte start address of second block (4)

263           Number of bytes in this block (128)

264 - 392     Program code for second block (second lot of data here)

393           Checksum for second block (whatever)

Okay, so this may not be the best description in the world, but hopefully you

get the general idea. There may be as many blocks of this type as you need,

and theycan all be any size from 1 to 256 bytes long. Once the file has been

completely loaded into the drive, execution starts from the start address of

the FIRST block be be transferred.

|> the total every time it exceeds 255. Thus the checksum byte can range from

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

|> 1-255.

^^^^^^

The checksum could be 0 if all the data bytes were 0 :-)

Seriously though, the point of having the checksum calculated in this way is to

make use of the carry bit in machine code addition if a carry is generated. The

point can be made more clearly in the following code fragment:

[accumulator holds current data byte]

clc        ;Clear carry bit for first add

adc $87    ;Add byte to current checksum

adc #$00   ;Add carry bit to checksum

sta $87    ;Save checksum to memory

If the sum of the accumulator and the contents of location $87 is greater than

#$FF then the carry flag will be set, in order to do multi-byte addition

correctly. Instead of adding this to a second byte however, it is added to the

same byte which generated the carry. This causes the value in the accumulator

to be incremented by 1 - the same as if #$FF had been subtracted from it.

If the sum is of the accumulator and the contents of location $87 is less than

or equal to #$FF, then no carry is generated and no further addition or

subtractionis needed.

|>      Below is a program which will make it easier for you to create a USR

|> file in the required format, so that it can be executed by the utility

|> loader command...

Here is a new version the the program, modified to handle multiple data blocks,

and a maximum program size of 256 bytes per block:

10  OPEN2,8,2,"0:&filename,U,W"

20  READNB

30  READLO,HI,LN:C=LO+HI:C=C+(C>255)*255

40  C=C+(LNAND255):C=C+(C>255)*255

50  PRINT#2,CHR$(LO);CHR$(HI);CHR$(LNAND255);

60  READD:PRINT#2,CHR$(D);

70  C=C+D:C=C+(C>255)*255

80  LN=LN-1:IFLN>0THEN60

90  PRINT#2,CHR$(C);

100 NB=NB-1:IFNB>0THEN30

110 CLOSE2

120 END

190 :

200 DATA2       :REM number of data blocks

210 DATA128,3   :REM lo/hi start address of first block

220 DATA6       :REM length of first block

230 :            REM program code

240 DATA32,71,198,76,0,3

300 DATA0,3     :REM lo/hi start address of second block

310 DATA26      :REM length of second block

320 :            REM rest of program code

330 DATA173,0,28,41,16,201,16,208,11

340 DATA169,247,45,0,28,141,0,28,76

350 DATA0,3,32,24,193,76,0,3

The example code here is just a minor extension to the original code. It

re-initializes the drive before jumping to the original code. This is just to

demonstrate that both parts of the code are being loaded and executed

correctly.

|> ERRORS THAT CAN OCCUR

|>

|> 51, 'overflow in record'

|>         Either: 1. The end of the file was reached before the checksum

|>                    byte was read.

|>         or      2. The file contains extra bytes of data after the checksum

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

|>                    byte.

^^^^^

|>         Check the length byte in the usr file (position 3, after lo/hi.)

The second condition is not quite accurate. If end-of-file is reached before

the drive expects to read the checksum, then this error will occur. If extra

bytes appear after the checksum, the drive assumes these to be the start of a

new block of data and will try to interpret these as address bytes, length

byte, data bytes etc as above. When end-of-file appears, the drive will report

this error, since it is expecting another block of data to appear and cannot

complete the transfer of this into memory. Hence the definition of this error

should be:

The end of the file was encountered unexpectedly. May indicate an

incorrect length byte, or additional data written after the end of the

last data block.

Note the semicolon at the end of line 90 of the program. If this were omitted,

a carriage return byte would be sent by the computer after the checksum byte

and may be the cause of this message.

One caution should be added at this point. Although data may be transferred to

any address in the RAM of the drive, it should be remembered that part of the

memory will be allocated to buffering the file as it comes off the disk. Thus

it may be possible to overwrite incoming data being buffered in memory before

it can be transferred correctly to its new location. Experimentation may be

the best way of determining which areas are "safe" and which ones have

problems. I found that the file seemed to be buffered in locations $0600 to

$0700, but this would depend on how may files you had open, etc.

Nick.

----

email: ncull@comp.vuw.ac.nz    Victoria University of Wellington, New Zealand.