This I/O map applies to c16/116/+4 computers

(c) 1990-1994 by Michael Stiller.

Extension port pinouts (50 pin female)

25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01

-- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

-- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

CC BB AA Z  Y  X  W  V  U  T  S  R  P  N  M  L  K  J  H  F  E  D  C  B  A

1       gnd                     A       gnd

2       + 5V                    B       C1 low

3       + 5V                    C       BRESET       --

4       IRQ       --            D       RAS          --

5       R/W                     E       I0  (clock0)

6       C1 high                 F       A15

7       C2 low                  H       A14

8       C2 high                 J       A13

9       CS1       --            K       A12

10      CS0       --            L       A11

11      CAS       --            M       A10

12      MUX                     N       A9

13      BA                      P       A8

14      D7                      R       A7

15      D6                      S       A6

16      D5                      T       A5

17      D4                      U       A4

18      D3                      V       A3

19      D2                      W       A2

20      D1                      X       A1

21      D0                      Y       A0

22      AEC                     Z       nc

23      extern Audio            AA      nc

24      02      (clock2)        BB      nc

25      gnd                     CC      gnd

Verified with the CBM Plus/4 User's Manual.

Note:pins marked with '--' appear in the schematic as low active

User port pinouts (24 pin male)

1  2  3  4  5  6  7  8  9  10 11 12

-- -- -- -- -- -- -- -- -- -- -- --

-- -- -- -- -- -- -- -- -- -- -- --

A  B  C  D  E  F  H  J  K  L  M  N

1       gnd                     A       gnd

2       +5V                     B       P0

3       BRESET   --             C       RxD

4       P2                      D       RTS

5       P3                      E       DTR

6       P4                      F       P7

7       P5                      H       DCD

8       RxC                     J       P6

9       ATN                     K       P1

10      9V AC                   L       DSR

11      9V AC                   M       TxD

12      gnd                     N       gnd

Verified with the CBM Plus/4 User's Manual.

Note:pins marked with '--' appear in the schematic as low active

These pinouts are from the book:

Wilhelm Besenthal/Jens Muus

'Alles ueber den Plus/4'

Markt&Technik Verlag AG

ISBN3-89090-410-6

Memory Map Overview

from the CBM Plus/4 User's Manual

Address         RAM             ROM

$ffff       End of RAM          ROM Bank High

$fffe                           IRQ Vector

$fffc                           RES Vector

$fffa                           NMI Vector

$ff81                           Kernal Jump Table

$fd00       I/O Register/TED Register

End if Basic RAM+1

$fc00                           ROM-Banking routines

$d800                           Operating System

$d000                           Character Table

$c000                           Basic extensions

$bfff                           ROM Bank Low

$8000                           BASIC

$4000       start of basic ram

if graphic

$3fff

graphic bitmap

$2000

color map (graphic)

$1c00

luminance (graphic)

$1800

$1000       start of basic ram

(no graphic)

$0c00       character bytes

(display)

$0800       attribute bytes

Detailed I/O Port Map as located in Plus/4 and similar CBM Computers

(C16/116/+4)

Most ports are not fully decoded by a chip called 7700-xx in the

schematic, so they may appear

copied on different memory locations.

Addr $       Chip/Port       Usage:

$fd00-fd03   ACIA 6551A      used for RS232  (for bits see +4 manual)

$fd00           DATA port

$fd01           STATUS port

$fd02           COMMAND port

$fd03           CONTROL port

$fd04-fd07   ACIA copy

$fd08-fd0b   ACIA copy

$fd0c-fd0f   ACIA copy

$fd10        PIO 6529B       User Port PIO (P0-P7)

Bit 2 (P2) is used to detect

if play on cassette is pressed

(CST sense)

$fd11-fd1f   PIO copies

$fd20-fd2f   free            not connected

$fd30        PIO 6529B       Keyboard PIO  Keyboard Matrix Connector

P0            15

P1            18

P2            7

P3            12

P4            11

P5            10

P6            5

P7            1

$fd31-fd3f   PIO copies

$fd40-fdcf   free            not connected

$fdd0-fddf   ADDR CLK        used for module select?

in the schematic this is

the clk/in for 4 D-FlipFlops

located in the 74LS175 chip which generates

the C1,C2 low/high signals

$fde0-fdff   free            not connected *

$fe00-fe1f   free            not connected *

$fe20-fe3f   free            not connected *

$fe40-fe5f   free            not connected *

$fe60-fe7f   free            not connected *

$fe80-fe9f   free            not connected *

$fea0-febf   free            not connected *

$fec0-fedf   TIA 6523A       if you have a CBM 1551 as device #9

then the TIA located in the paddle of

you drive will appear at this location

for details see $fee0-feff

$fee0-feff   TIA 6523A       if you have a CBM 1551 as device #8

then the TIA located in the paddle of

your drive will appear at this location

$fee0           TIA PORT A   (DATA)

$fee1           TIA PORT B   (STATUS)

$fee2           TIA PORT C   (HANDSHAKE)

$fee3           TIA PORT A   data direction register

$fee4           TIA PORT B   data direction register

$fee5           TIA PORT C   data direction register

$fee6           not connected

$fee7           not connected

$fee8-feef      TIA copy

$fef0-fef7      TIA copy

$fef8-feff      TIA copy

TIA Notes:

The TIA 6523A is a triport interface adapter which is located in the

paddle of the 1551 disk drives. The drivenumber causes the TIA to appear

at the corresponding memory addresses as mentioned above.

The address decoding stuff is done by another chip located in the

1551 paddle, called 8700-011 ?

The TIA port a is used as direct 8 bit data connection between the

computer and the drive. (It is called TCBM Bus in the 1551 schematic)

The other TIA ports are used for other TCBM Bus signals (handshake etc.)

They are connected as follows:

Port    bit     used for

B       0       status 0

B       1       status 1

C       6       DAV signal

C       7       ACK signal

The marked (*) port may be intended as additional drive locations,but

they are not supported by the 1551 drives.

(c) 1990-1994 by Michael Stiller.

The informations here are given without warranty.

May not be modified without permission.

For comments,questions,donations etc. write to

stille00@marvin.informatik.uni-dortmund.de