From: brain@garnet.msen.com (Jim Brain)

Newsgroups: comp.sys.cbm

Subject: PC's as 64 HD's and the 6522 chip!

Date: 7 Apr 1994 13:48:35 GMT

Right after I posted about hooking a 6522 VIA up to a PC parallel port

to turn the PC into a "char-banger" instead of a "bit-banger", someone

posted that they had thought the 6522 had a problem with the shift

register.  The poster also said that Jim Butterfield had alerted him and

others to it.

Well, I have been in correspondence with "The Commodore Man", so I asked him

to elaborate on the topic.  Here is the two responses:

>  Yes, it's true.  Although I didn't get official confirmation of this

>long after, when a Spectrum article quoted the designers.

>  As you know, the first Commodore computers used the IEEE bus to connect to

>peripherals such as disk and printer.  I understand that these were available

>only from one source:  Belden cables.  A couple of years into Commodore's

>computer career, Belden went out of stock on such cables (military contract?

>who knows?).  In any case, Commodore were in quite a fix:  they made

>computers and disk drives, but couldn't hook 'em together!

>  So Tramiel issued the order:  "On our next computer, get off that bus.

>Make it a cable anyone can manufacture".  And so, starting with the VIC-20

>the serial bus was born.  It was intended to be just as fast as the

>IEEE-488 it replaced.

>  Technically, the idea was sound:  the 6522 VIA chip has a "shift

>register" circuit that, if tickled with the right signals (data and clock)

>will cheerfully collect 8 bits of data without any help from the CPU.

>At that time, it would signal that it had a byte to be collected, and

>the processor would do so, using an automatic handshake built into the

>6522 to trigger the next incoming byte.  Things worked in a similar way

>outgoing from the computer, too.

>  We early PET/CBM freaks knew, from playing music, that there was something

>wrong with the 6522's shift register:  it interfered with other functions.

>The rule was:  turn off the music before you start the tape!  (The shift

>register was a popular sound generator).  But the Commodore engineers,

>who only made the chip, didn't know this.  Until they got into final

>checkout of the VIC-20.

>  By this time, the VIC-20 board was in manufacture.  A new chip could

>be designed in a few months (yes, the silicon guys had application notes

>about the problem, long since), but it was TOO LATE!

>  A major software rewrite had to take place that changed the VIC-20

>into a "bit-catcher" rather than a "character-catcher".  It called for

>eight times as much work on the part of the CPU; and unlike the shift

>register plan, there was no timing/handshake slack time.  The whole

>thing slowed down by a factor of approximately 5 to 6.

>  There's more (the follow-on C64 catastrophe), but that's where it

>happened.

--Jim

And the saga continues ...

>  When the 64 came out, the problem VIA 6522 chip had been

>replaced by the CIA 6526.  This did not have the shift register problem

>which had caused trouble on the VIC-20, and at that time it would have

>been possible to restore plan 1, a fast serial bus.  Note that this would

>have called for a redesign of the 1540 disk drive, which also used a VIA.

>   As best I can estimate - and an article in the IEEE Spectrum magazine

>supports this - the matter was discussed within Commodore, and it was

>decided that VIC-20 compatibility was more important than disk speed.

>Perhaps the prospect of a 1541 redesign was an important part of the

>decision, since current inventories needed to be taken into account.

>   But to keep the Commodore 64 as a "bit-banger", a new problem arose.

>The higher-resolution screen of the 64 (as compared to the VIC-20)

>could not be supported without stopping the CPU every once in a while.

>To be exact:  Every 8 screen raster lines (each line of text), the CPU

>had to be put into a WAIT condition for 42 microseconds, so as to allow

>the next line of screen text and color nybbles to be swept into the chip.

>(More time would be needed if sprites were being used).

>   But the bits were coming in on the serial bus faster than that:  aD

>a bit would come in about every 20 microseconds!  So the poor CPU, frozen

>for longer than that, would miss some serial bits completely!

>   Commodore's solution was to slow down the serial bus even more.

>That's why the VIC-20 has a faster serial bus than the 64, even though

>the 64 was capable, technically, of running many times faster.

>   Fast disk finally came into its own with the Commodore 128.

--Jim

Now someone also told me at one time that they had seen a fastloader that

same someone said he thought it was odd that the author of the loader had

credited Commodore with the routines.  Well, I can hazard a guess that the

routines were the ones they had wanted to put in the 6522, but had to scrap

due to the 6522 problem.  Now I have no idea what the problem is/was, but

I am eager to find out.  However, I rescind my plans to build something

around the 6522 until we find out what the problem is.

Jim "Just the Facts" Brain

--

Jim Brain, Embedded Systems Designer, Brain Innovations.

brain@msen.com

Dabbling in VR, Old Commodore Computers, and Good Times!

"The above views DO reflect my employer, since I am my employer" - Jim Brain

--

From: dillon@apollo.west.oic.com (Matthew Dillon)

Newsgroups: comp.sys.amiga.advocacy,comp.sys.cbm

Subject: Re: COMMODORE STOCK SPECULATION!!

Date: 8 Apr 1994 23:15:06 -0700

In article <1994Apr8.020037.9386@pimacc.pima.edu> ppugliese@pimacc.pima.edu writes:

:In article <2o1pjq$20a@netaxs.com>, firefoot@netaxs.com (Ivan Kohler) writes:

:> ppugliese@pimacc.pima.edu wrote:

:> : In article <2ns7bt$ivu@falcon.ccs.uwo.ca>, koops@gaul.csd.uwo.ca (Luke

:>  Koops) writes:

:> : > In article <1994Apr3.181524.821@mnemosyne.cs.du.edu>,

:

:> : >

:> : > Grapevine has made KS chips and 8372A's. If you are in good enough with

:> : > Commodore, you can make your own chips with an eprom burner. We used to do

:...

:

:

:No. I was only referring to the chips that the previous poster mentioned.

:The custom ships would have to be fabricated, although the CIA may not

:be so hard to find, after all. I say that because I once was looking for

: the VIA chips that the 1541 uses & found them "on the rack" in an elec-

: tronics supply house. Some guy in the LUG said that those chips were used

: for "alot of things".

Most of Commodore's older chips are also made by Rockwell, but

unfortunately they haven't fixed any of the bugs.  For example,

the 65C22 still has a number of interrupt/timer race conditions.

I don't quite remember the details of all the bugs, but I found

on the order of 3 hardware bugs in the 65C22.  The chips used in

the C64 also have problems... Bryce found at least one hardware

race condition that would screw up a timer interrupt.

I do not know whether any of Commodore's newer chips have secondary

sources... certainly the big custom chips do NOT.

But even if C='s chip fab goes out of business, they could easily

license the schematics to a professional workshop.  NRE would probably

be around $50,000 to reimplement them with a more modern process

(Commodore was using 5 micron NMOS last I heard, or is it 3 now?).

-Matt

--

Matthew Dillon		dillon@apollo.west.oic.com

1005 Apollo Way

Incline Village, NV. 89451	ham: KC6LVW (no mail drop)

USA 			Sandel-Avery Engineering (702)831-8000

[always include a portion of the original email in any response!]