From: Raymond Carlsen <rrcc@u.washington.edu>

Newsgroups: comp.sys.cbm

Subject: Heat sinks for ICs in CBM computers

Date: Tue, 4 Apr 1995 19:28:31 -0700

As I promised last week, here is the article regarding adding heat sinking

to the IC chips in a C-64. (Sorry it took so long to post this. I am new

on this system and no one around here can answer my questions. I had to

figure this out for myself... sure hope it works this time!   Ray C.)

Solid state devices such as Integrated Circuits (ICs) fail for two

basic reasons, assuming they were not defective from the factory...  1.

Overvoltage, either from a defective power supply or a "spike" on one of

the lines in or out of the device, and 2. Excessive heat, which exceeds

the device limits. Long-term temperature cycling also leads to breakdown

after an unpredictable amount of time. The causes of overvoltage by way of

the power supply are almost predictable... the Commodore "black brick" has

a history of catastrophic failure, sometimes taking the computer with it.

After-market repairable supplies are much more reliable because they are

built with higher current capability than the original. Spikes (static

electricity or voltage surges) can destroy ICs in the computer through one

of the "accessory" ports. Both of these hazzards are preventable to a

degree... the use of a higher quality supply and simple precautions such

as never plugging anything in (cartridges, mouse, etc.) when the computer

is turned on just make good sense. Static damage can be kept to a minimum

by grounding everything that will be used with the computer before

connecting it.

A less predictable but no less important concern is the fact that

repeated heating and cooling of semiconductor devices (especially ICs and

power transistors/diodes) under normal use, even within their "safe"

temperature range, will eventually cause them to fail. It is usually the

devices that generate the most heat in normal operation that fail first.

It's a bit like bending a piece of metal back and forth until it fatagues

and breaks. Repeated heating and cooling does the same thing to the

structure of the microscopic components inside an IC or a transistor.

You could leave the computer on 24 hours a day... some systems run

that way. A BBS system is a good example. I know of one system that has

run without a breakdown for years (except for the hard drive power

supplies). For most of us, however, the computer doesn't get enough use to

justify the expense of a system running all the time. An alternate

solution is to reduce the extremes of temperature inside the ICs by use of

cooling fans and/or heat "sinks", and thereby reduce the thermal stress on

the devices.

Microprocessors in new IBM type computers are running so fast

nowadays (60 to 100MHz) that heat sinking and direct fan cooling is

absolutely necessary... those chips would overheat and fail in a very

short time without some method to dissipate that heat! Some of the

Commodore 64 ICs run too hot to touch in normal operation. There are two

simple modifications you can do to improve the reliability of your C-64,

namely, increasing the airflow inside the case, and heat-sinking the ICs.

The physical design of the C-64 limits the airflow over the chips.

Tiny air vents in several places in the upper and lower half-shells offer

minimal convection cooling. That cardboard and tinfoil shield, although of

questionable usefullness as an interference filter, is great at blocking

the airflow and holding all the heat inside the case. I have run tests to

see how effective that shield is in reducing radio and TV interference. I

found little difference after removing the shield. In fact, the 1541 (with

all shields in place) generates more "noise" than the C-64. Without the

shield, the C-64 might be more prone to pick up noise from external

sources such as a monitor or TV, but I have never found that to be a

problem (none of my computers has the shield anymore). The increased

airflow without the shield inside the C-64 case helps to keep the chips

cooler.

Three of the large ICs tend to run hot: the PLA, the SID and the VIC.

These are the ones that fail most often and would benefit most from "heat

sinking"... drawing away the normally produced waste heat from the device

by means of a metal "fin" attached to the case of the IC. The C-64C (flat

case) and the C-128 have a shield plate with "fingers" that touch each of

the larger ICs and dissipate some of their heat into the surrounding

air. The SX-64 has individual heat sinks glued to the larger chips. Most

C-64s had no such sinking.

Remove the screws from the bottom of the computer, but leave the

keyboard in place, then run the computer for half an hour, lift the

keyboard and feel the tops of all the chips... some will be quite hot!

Those are the ones that most need the help. You can purchase commercial

heat sinks that fasten to the top of the chips with clamps, adhesive

strips or glue. Radio Shack and other parts houses can supply them in a

variety of sizes. If you want to make your own, any thin metal stock will

do. The only consideration is that it be flexable enough to bend without

breaking, and stiff enough to hold its' shape when bent... it must be flat

on the bottom for intimate contact when glued to the chip. I use strips of

"tin" cut from large juice cans. With tin snips, cut a piece of metal

about 4" long and about 1.5" wide from the "un-ribbed" part of the can,

and bend it to fit the top of the IC. Careful! Those cut ends are razor

sharp! The top of a 28 pin chip such as the PLA is approximately 1.5" long

and 5/8" wide. The heat sink is fashioned into a "U" shape with "ears"

that extend out to either side. The sink ears are made high enough to

clear all components on the PC board, but not touching the keyboard when

it is reattached: bend the ears over about 1/2" from the top of the IC.

The large "surface area" is necessary since the airflow inside the case is

minimal to begin with.

SIDE VIEW OF HEAT SINK ATTACHED TO IC

|<-------------- APPROX 3" --------------------->|

~~~~~~~~~~~~~~~~~|             |~~~~~~~~~~~~~~~~~~

.5" /\ |<---5/8"---->|<----- HEAT SINK

\/   |_____________|

----  =============== <------ ADHESIVE

|~~~~~~~~~~~~~|

|______|______|<----IC

___________________!________!______________<----CIRCUIT

!        !                    BOARD

Use fast-set epoxy (15 min) to attach the sink to the IC. If you have

several sinks prepared ahead of time, you can mix one batch and glue them

all before the epoxy starts to set. Print the name or number on the top of

the sink with a felt tip pen so you can still identify the chips when

you're done. Be careful not to use too much glue (one small pea-sized drop

is plenty), or it will ooze out the sides and down into the socket. (Note:

not all chips are socketed.) Spread the epoxy over the sink (or on top of

the chip) and gently press it into place. I don't recommend super glue as

the surface of the metal will not be flat enough for a good bond. Heat

sink compound (white silicon "goo") is sticky enough if the computer will

never be moved, but I don't recommend it either. If the metal sink falls

into the computer, it will most certainly short something out. Epoxy forms

a good bond even with irregular surfaces as it fills the "voids" and

provides sufficient heat transfer to the sink. Just make sure the metal is

free of dirt, oil, fingerprints, etc.

As I stated, the chips that benefit most from sinking are the large

heat producing ones, like the PLA (CBM #906114-XX, 82S100PLA, or 93459),

the SID (CBM #906112-01 or 6581), and the VIC (CBM #906109 or 6567). The

VIC chip is already sinked in later C-64s. It's inside a metal box... the

cover has a tab that touches the VIC chip. If yours doesn't have one, sink

it. You can use the "finger" test to see if any other chips in your

computer run hot enough to need sinking (too hot to touch after 1/2 an

hour with the case closed). The only other chip I would sink would be the

MPU (6510), although I have only seen a few bad ones. Make sure the heat

sink fins don't touch any circuit components and cause a short!

The two VIA (6526) chips run cool to the touch and don't really need

sinking. Since they connect to the external ports, they are more sensitive

to static "zaps". The ROMs run warm but seem to last forever anyway...

same with the eight RAM chips (4164), at least until the power supply

regulator fails... when RAM chips get hot to the touch, they're gone!

Normally, they run cool.

Simalar modifications can be made to the 1541 disk drive. It's power

supply is internal, making the IC heat problem even worse than in the

computer. The problem with adding heat sinking to the drive is the space

limitations inside the case... it's pretty crowded in there, but it can be

done. Find the chips that get the hottest, and make the necessary

modifications. The heat sinks you make will have to be cut to fit the

space allowed. With these simple modifications, you can keep your little

'64 system humming along for many more years. :-)

Ray Carlsen   CET